Notes on the Trizeps VII iPAN5 Eval-Kit

This evaluation-kit boots a preliminary version of a Windows Embedded Compact 7 image.

The image is booted from a uSD-card inserted in the uSD-card-slot of the Trizeps VII module.

To create a new SD-card copy the contents of the uSD-card to a directory on your PC and call FormatBurnOS_***.bat. You have to mount the uSD-card as drive S: or you have to change the drive-letter in the batches.

You have to choose the following batch:

- FormatBurnOS_quad.bat 						-> i.MX6 quad/dual core

- FormatBurnOS_solo_Memphis_32Bit_1GByte_MEM4G16D3E.bat 	-> i.MX6 solo/dual-lite core and 1GByte Memphis-RAM

- FormatBurnOS_solo_NANYA_32Bit_512MByte_NT5CB128M16FP.bat 	-> i.MX6 solo/dual-lite core and 512MByte

If you use a with cfimager.exe formated uSD-card you can use the BurnOS_***.bat to burn only the image without formatting.

To use the samples you have to copy them to the uSD-card. They can be found at wince under “\SDMMC Card”.

Sample-Applications can be found in the \Demos\ -folder:

.\Cube				- shows a rotating 3D cube.

.\GCTiger			- shows a rotating 2D image.

.\Storyboard			- Storyboard demo from Crank (www.cranksoftware.com)

Please run the links in the .\storyboard –root-folder.

.\HTML5			- HTML5-engine from BSquare (www.bsquare.com).

bsqBrowser.exe	  - HTML5-browser

bsqShell.exe		  - Shell based on HTML5. Similiar to Windows Phone design.

Please note that some applications need the 2D/3D GPU enabled. It gets disables by .\update.exe on start to save power. You can reenable these units by running the “GPU Clock” application.

You can use the image with the iPAN5-Display or HDMI-Display with FULL-HD or XGA resolution.

To use HDMI-Display you must rename the file hdmi_fullhd.png or hdmi_xga.png to boot.png.

Please note that the HDMI-Display not display the boot-display and you have to escape the touchscreen initialization.